# US-1000 Ultrasonic Distance Sensor

# Accurate Distance Measurement in Real-Time

## Section 1: General Intro, Features, and Specifications

### Introduction

The US-1000 Ultrasonic Distance Sensor is a compact and precise device designed for measuring distances using ultrasonic sound waves. It is ideal for robotics, automation, and object detection applications.

### Key Features

- Non-contact distance measurement

- Real-time output via serial interface

- Range: 2 cm to 400 cm

- Operating Voltage: 5V DC

- Compact PCB with onboard microcontroller

- LED indicator for power and activity status

### Specifications

#### Physical:

- Dimensions: 6.35cm x 5.08cm

- Screw holes for attachable legs

#### Electrical:

- Supply Voltage: 5V DC

- Current Consumption: ~60mA at 100% use

#### Software:

- Embedded C code for HC-SR04 sensor control

- Ready Built main, header, and function files

- Output: Distance in centimeters

## Section 2: Installation and Operating Instructions

### Setup Instructions (Hardware)

- Plug the US-1000 into the arduino pins on the STM32 microcontroller located in the middle of the Base Station (NO FLOATING PINS).

- Snap the “eye” board into the plastic mount.

- Using a 4 wire FF ribbon cable connect the “eye” board to the “+” and “-” pins (Be sure to connect properly).

- Screw the plastic mount onto a stationary or servo stand.

### Setup Instructions (Software)

- Download these .txt files.

- Copy the code into STM32Cube IDE or a compatible software.

- Run the code and verify the OLED is displaying the distance.

- If the OLED is displaying an incorrect distance modify the distance equation in the functions file to the proper speed of sound for your application.

## Section 3: Theory of Operation

The US-1000 operates by emitting 8 40kHz ultrasonic pulses via TIM2. These pulses reflect off nearby objects and are received by pin A4. The time interval between sending and receiving is calculated by the onboard microcontroller using TIM14 to determine the distance:

distance = (34300.0584 x (EchoTime) / 1000000) / 2

The microcontroller then displays this distance on an OLED screen.

## Section 4: Appendices

### Mechanical Drawings

Robo Car Application

Mount

### Electrical Drawings

- Schematic diagram (Base Station)

- Schematic diagram (My System)

- PCB layout

### Software Listings

- STMCube IDE

### Data Sheets

- Base Station Cite: https://www.vtelabs.com/stm32

- MAX232 Datasheet: https://www.ti.com/lit/ds/symlink/max232.pdf?ts=1746530299319

- TL084 Datasheet: https://www.ti.com/lit/ds/symlink/tl084.pdf?ts=1746558136866&ref_url=https%253A%252F%252Fwww.ti.com%252Fproduct%252FTL084%252Fpart-details%252FTL084CN

### Bibliography

- HC-SR04 Datasheet: https://cdn.sparkfun.com/datasheets/Sensors/Proximity/HCSR04.pdf

- David Pilling: https://www.davidpilling.com/wiki/index.php/HCSR04